ASTM- Uniformat II - Intended configuration model

I want to use the full Uniformat workbook as the master building-element database, then create configurable assessment profiles for different service lines.

The proposed logic is:

- Master taxonomy

  - Retain the complete hierarchy from Levels 1–5.

  - Do not permanently remove codes from the database.

  - Treat ASTM E1557 / Uniformat II as the common classification backbone.

- Implementation configuration

  - Create a standard configuration for Building Condition Assessments.

  - Create a separate standard configuration for Building Envelope Assessments.

  - For each configuration, activate or deactivate:

    - Level 1 groups;

    - Level 2 systems;

    - Level 3 elements;

    - Level 4 sub-elements;

    - Level 5 internal extensions.

- Project-level configuration

  - When a new assessment is created, the PM receives the applicable standard configuration as a starting point.

  - The PM can then include or exclude individual systems and components based on:

    - scope;

    - building type;

    - client requirements;

    - accessibility;

    - assessment type;

    - exclusions in the proposal.

- Operational result

  - Only the activated codes appear in:

    - intake;

    - inspection checklists;

    - analyst workspaces;

    - costing;

    - report generation;

    - QA;

    - exports.

That is a sound architecture. It gives you a controlled master standard without forcing every project to inspect every possible building component.

One item appears unrelated to this use case: the paragraph referring to electricity-system planners, utilities, ISOs and transmission companies. I assume that was pasted from another product description and is not the target market for this BCA and building-envelope solution.

Recommended configuration approach

1. Maintain one complete master taxonomy

The workbook should become the Master Component Catalogue.

Each record should contain at least:

| Field | Purpose |
| --- | --- |
| Code | Unique Uniformat or internal component code |
| Title | Component name |
| Level | Level 1–5 |
| Parent code | Maintains hierarchy |
| Full path | Shows complete lineage |
| Source type | ASTM, suggested Level 4, or internal Level 5 extension |
| Active globally | Whether the code is available for configuration |
| Assessment applicability | BCA, Envelope, both, optional, excluded |
| Default inspection status | Included, optional, excluded |
| Default inspection depth | Screening, standard, detailed, specialist |
| Allowed child selection | Whether PM may activate/deactivate children |
| Cost-enabled | Whether quantity and unit-rate fields apply |
| Photo-required | Whether field evidence is normally mandatory |
| Drawing-enabled | Whether drawings or markup are expected |
| Report section mapping | Where the record appears in outputs |

A critical governance point is that ASTM E1557 formally defines Levels 1–3 and provides a suggested Level 4. Your Level 5 content should therefore be treated as a controlled internal or industry-derived extension, rather than described as universally standardized ASTM Level 5.

2. Recommended default BCA configuration

A BCA is broad. Its normal starting point should be Level 3, because ABSI has confirmed that Level 3 is generally appropriate for BCA work.

The applicable scope should normally span the main building and site systems.

BCA Level 1 applicability

| Level 1 | Default status | Recommendation |
| --- | --- | --- |
| A – Substructure | Active | Core BCA scope |
| B – Shell | Active | Core BCA scope |
| C – Interiors | Active | Core BCA scope |
| D – Services | Active | Core BCA scope |
| E – Equipment & Furnishings | Optional | Activate only where owner-maintained or materially relevant |
| F – Special Construction | Optional | Activate for special structures, facilities or hazardous components |
| G – Building Sitework | Active | Core BCA scope where site is included |
| H – General | Administrative only | Use for reports, surveys, contingencies and project controls—not physical inspection components |

ASTM E2018 describes a baseline property-condition assessment process but expressly allows the scope and level of due diligence to vary according to the user’s objectives, risk tolerance, schedule and budget. That supports your proposed configuration model rather than a single fixed checklist.

BCA Level 2 and Level 3 defaults

A — Substructure

Default active:

- A10 Foundations

  - A1010 Standard Wall Foundations

  - A1020 Special Foundations

  - A1030 Slab on Grade

- A20 Basement Construction

  - A2010 Basement Excavation — usually contextual or optional

  - A2020 Basement Walls

  - A2030 Structural Columns/Beams

Recommended depth:

- Level 3 default

- Level 4 or 5 only where deterioration, garage structures, waterproofing or specialist structural review requires additional granularity

B — Shell

Default active:

- B10 Superstructure

  - B1010 Floor Construction

  - B1020 Roof Construction

  - B1030 Bearing Structure

- B20 Exterior Closure

  - B2010 Exterior Walls

  - B2020 Exterior Windows

  - B2030 Exterior Doors

- B30 Roofing

  - B3010 Roof Coverings

  - B3020 Roof Openings

Recommended depth:

- Level 3 for standard BCA

- Level 4 or 5 for components requiring costing, observed deficiencies or envelope coordination

C — Interiors

Default active:

- C10 Interior Construction

  - C1010 Partitions

  - C1020 Interior Doors

  - C1030 Fittings

- C20 Staircases

  - C2010 Stair Construction

  - C2020 Stair Finishes

- C30 Interior Finishes

  - C3010 Interior Wall Finishes

  - C3020 Floor Finishes

  - C3030 Ceiling Finishes

Recommended depth:

- Level 3 is generally sufficient

- PM may deactivate tenant-owned or excluded interior finishes

D — Services

Default active:

- D10 Conveying Systems

- D20 Plumbing

- D30 HVAC

- D40 Fire Protection

- D50 Electrical

All relevant Level 3 children should remain available.

Recommended depth:

- Level 3 default

- Level 4 or 5 only where the service scope requires equipment-level inspection and costing

E — Equipment & Furnishings

Default:

- Optional, not automatically included

Activate when:

- the equipment is owner-owned;

- it is part of the BCA scope;

- failure materially affects operations;

- replacement costs are required.

F — Special Construction

Default:

- Optional

Activate when the property contains:

- special structures;

- integrated construction systems;

- special facilities;

- demolition or abatement scope.

G — Building Sitework

Default active where sitework is included:

- G20 Site Improvements

  - G2010 Roadways

  - G2020 Parking Lots

  - G2030 Pedestrian Paving

  - G2040 Site Development

  - G2050 Landscaping

- G30 Site Civil / Mechanical Utilities

- G40 Site Electrical Utilities

- G90 Other Site Items

G10 Site Preparation would normally be optional for an existing-building assessment because several of its entries are more relevant to construction or remediation than routine BCA inspection.

H — General

Use for workflow and reporting administration rather than field asset inspection:

- H1010 General Requirements

- H2010 Contingency

- H3010 Reports

- H4010 Survey

I would not place these codes in the inspector’s normal component checklist.

3. Recommended Building Envelope configuration

Building-envelope work is narrower in system breadth but deeper in component detail.

Its normal starting point should be:

- Level 3 to define the system;

- Level 4 for the standard component breakdown;

- Level 5 where ABSI requires material, assembly, location or defect-specific detail.

Envelope Level 1 applicability

| Level 1 | Default status | Recommendation |
| --- | --- | --- |
| A – Substructure | Selective | Include below-grade enclosure and waterproofing-related elements |
| B – Shell | Fully active | Primary envelope taxonomy |
| C – Interiors | Selective | Include interior evidence and finishes affected by envelope failures |
| D – Services | Selective | Include interfaces such as drainage, penetrations and roof-mounted systems |
| E – Equipment & Furnishings | Normally excluded | Activate only for direct enclosure interfaces |
| F – Special Construction | Optional | Include special envelope systems or structures |
| G – Sitework | Selective | Include drainage, grading and interfaces that affect the envelope |
| H – General | Administrative | Drawings, surveys, reports and project controls |

Core envelope codes

A — Substructure, selective activation

Recommended active:

- A1010 Standard Wall Foundations

- A1030 Slab on Grade, where under-slab moisture or perimeter interfaces are relevant

- A2020 Basement Walls

- selected foundation drainage, waterproofing and insulation Level 4–5 entries

Possible project-level additions:

- A2030 Structural Columns/Beams where deterioration affects enclosure performance

- garage walls, below-grade joints and buried waterproofing components

B — Shell, fully active

This is the core envelope group.

Recommended active:

- B1010 Floor Construction

  - balconies;

  - podium slabs;

  - exposed floor edges;

  - ramps and exterior structural interfaces.

- B1020 Roof Construction

- B1030 Bearing Structure where tied to enclosure support or distress

- B2010 Exterior Walls

- B2020 Exterior Windows

- B2030 Exterior Doors

- B3010 Roof Coverings

- B3020 Roof Openings

For envelope assessments, activate Level 4–5 detail for:

- wall assemblies;

- cladding types;

- masonry;

- sealants and joints;

- air barriers;

- vapour retarders;

- insulation;

- flashings;

- membranes;

- windows;

- glazing;

- perimeter seals;

- doors;

- balcony interfaces;

- parapets;

- roofs;

- drains;

- curbs;

- skylights;

- penetrations;

- transitions and terminations.

ASTM E2841 may also be relevant to facade-inspection methodology when the purpose includes identifying unsafe facade conditions. It addresses facade components, performance history and interaction between adjacent enclosure components.

C — Interiors, selective activation

Recommended optional:

- C3010 Interior Wall Finishes

- C3020 Floor Finishes

- C3030 Ceiling Finishes

These should not be treated as primary envelope assets. They are useful as evidence locations, for example:

- staining;

- efflorescence;

- mould indicators;

- peeling finishes;

- condensation damage;

- leakage paths.

The configuration should mark them as:

Interior manifestation of envelope condition

rather than classifying them as the root enclosure deficiency.

D — Services, selective activation

Recommended selective:

- D2040 Rain Water Discharge

- relevant roof drainage and internal rainwater components

- penetrations through the enclosure

- exhausts, louvers and intake openings

- roof-mounted mechanical interfaces

- electrical penetrations and exterior fixtures where relevant

The platform should distinguish:

- service-system condition;

- envelope interface condition.

G — Sitework, selective activation

Recommended selective:

- G1030 Site Earthwork, where grading affects water entry

- G2030 Pedestrian Paving

- G2040 Site Development

- G3030 Storm Sewer

- relevant drainage and foundation-interface elements

This permits envelope findings to capture:

- negative grading;

- drainage toward the building;

- failed paving-to-wall joints;

- foundation drainage issues;

- splashback and snow accumulation;

- surface-water management.

4. Do not configure only by “level”

This is an important design principle.

A Level 5 code is not automatically more relevant than a Level 3 code. Relevance is based on:

- assessment type;

- scope;

- component presence;

- required output;

- inspection depth;

- cost-planning requirements.

Therefore, configuration should operate at the individual code level, with optional hierarchy-level controls.

For example:

- Activate B20 at Level 2.

- By default, inherit all B2010, B2020 and B2030 children.

- Deactivate B2030 if exterior doors are excluded.

- Expand B2020 to Level 5 if the project requires window-by-window assessment.

- Keep B2010 at Level 4 if the client only needs wall-system reporting.

5. Recommended configuration hierarchy

Use four configuration layers.

Layer 1 — Master taxonomy

The complete controlled Uniformat library.

No project user deletes from this layer.

Layer 2 — Service-line template

Examples:

- BCA — Standard

- BCA — Limited

- BCA — Comprehensive

- Building Envelope — Visual

- Building Envelope — Detailed

- Facade Safety Assessment

- Roof Assessment

- Garage Condition Assessment

Each template defines the default activated codes and depth.

Layer 3 — Organization or client template

Allows ABSI or a major client to specify:

- preferred codes;

- terminology;

- report format;

- condition scale;

- cost fields;

- mandatory photos;

- report sections;

- exclusions.

Layer 4 — Project assessment scope

The PM creates the final project-specific selection.

The project should inherit defaults but permit controlled overrides.

6. Recommended activation states

Do not use only a simple yes/no flag.

Use the following states:

| State | Meaning |
| --- | --- |
| Required | Always included and cannot be removed without approval |
| Default Included | Automatically selected but PM may remove |
| Optional | Available for selection but not preselected |
| Conditional | Activated when a rule is met |
| Excluded | Not applicable to the assessment type |
| Not Present | Confirmed absent at this building |
| Out of Scope | Present but contractually excluded |
| Inaccessible | In scope but could not be inspected |
| Deferred | To be inspected in a later phase |

This distinction is essential for defensibility. “Excluded,” “not present,” and “inaccessible” are not equivalent.

7. Configuration rules and inheritance

The recommended inheritance model is:

Master taxonomy → service template → client template → project scope → field confirmation

Each lower layer inherits from the one above.

A PM override should not change the parent template.

Example:

- BCA Standard includes D10 Elevators.

- A particular low-rise building has no elevator.

- PM marks D10 as Not Present.

- D10 disappears from the inspection checklist for that project.

- The BCA Standard template remains unchanged.

8. Parent-child activation logic

Recommended rules:

Activating a parent

When a parent code is activated:

- children inherit the parent status;

- the PM may selectively deactivate children where permitted.

Deactivating a parent

When a parent is deactivated:

- all children become inactive;

- existing assessment data must trigger a warning before deactivation.

Activating a child under an inactive parent

The platform should:

- automatically activate all ancestors; or

- warn the user and request confirmation.

Level-depth control

Each activated branch should have:

- minimum reporting level;

- maximum selectable level;

- inspection depth.

Example:

| Branch | Minimum | Default | Maximum |
| --- | --- | --- | --- |
| BCA — HVAC | Level 3 | Level 3 | Level 4 |
| BCA — Roofing | Level 3 | Level 3 | Level 5 |
| Envelope — Exterior Walls | Level 3 | Level 4 | Level 5 |
| Envelope — Windows | Level 3 | Level 4 | Level 5 |

9. PM assessment-creation workflow

When the PM creates a new assessment:

- Select service type:

  - BCA;

  - building envelope;

  - roof;

  - facade;

  - garage;

  - custom.

- Select assessment depth:

  - limited;

  - standard;

  - comprehensive;

  - intrusive/specialist.

- Select building type:

  - residential;

  - commercial;

  - institutional;

  - industrial;

  - mixed use;

  - other.

- Load the recommended configuration.

- Review Level 1 and Level 2 scope.

- Expand relevant branches to Levels 3–5.

- Identify:

  - present;

  - not present;

  - excluded;

  - optional;

  - inaccessible or unknown.

- Select required output formats:

  - Excel;

  - Word;

  - PDF;

  - drawings;

  - photo log;

  - capital plan.

- Confirm costing requirements:

  - quantity;

  - unit;

  - unit rate;

  - escalation;

  - contingency;

  - timing.

- Freeze and issue the inspection scope.

The system then generates:

- field checklists;

- inspection navigation;

- report headings;

- costing lines;

- QA completeness rules.

10. Separate taxonomy from assessment logic

Do not overload the Uniformat code itself with every business rule.

Use separate relational structures:

Component taxonomy table

Defines what the component is.

Assessment profile table

Defines whether it is relevant for BCA, envelope or another service.

Project scope table

Defines whether it is included in the specific project.

Observation table

Stores what was seen.

Analyst decision table

Stores condition, deficiency, risk, priority, RUL and recommendation.

Output mapping table

Defines where the data appears in Word, Excel, PDF, drawings and dashboards.

This prevents the database from becoming rigid.

11. Suggested database entities

A practical configuration model could include:

| Entity | Purpose |
| --- | --- |
| ComponentCode | Master Level 1–5 hierarchy |
| AssessmentType | BCA, envelope, roof, facade, etc. |
| AssessmentTemplate | Named configuration profile |
| TemplateComponent | Code activation and depth by template |
| ClientTemplate | Client-specific overrides |
| ProjectAssessment | Individual assessment instance |
| ProjectComponentScope | Project-level include/exclude state |
| ComponentLocation | Building, elevation, room, zone, roof area |
| Observation | Field evidence |
| Deficiency | Analyst interpretation |
| Recommendation | Required action |
| CostItem | Quantity, unit, rate and timing |
| ReportMapping | Output location and presentation rules |
| ConfigurationAudit | Tracks who changed what and when |

12. Recommended MVP configuration

For the first implementation, avoid trying to perfect all 922 codes.

Start with:

BCA MVP

- Levels 1–3 across A–G

- selective Level 4 and 5 for:

  - roofing;

  - walls;

  - windows;

  - doors;

  - balconies;

  - garage components;

  - major mechanical/electrical equipment;

  - site paving and drainage.

Envelope MVP

- Level 3 for system navigation

- Level 4–5 for:

  - exterior walls;

  - windows;

  - exterior doors;

  - roofs;

  - roof openings;

  - balconies;

  - below-grade enclosure;

  - drainage and critical interfaces.

Administrative

- H codes should support project controls and report generation but remain outside the physical inspection tree.

My overall recommendation

Build the system around this principle:

One complete taxonomy, multiple controlled service configurations, and project-level scope refinement.

Do not create separate copies of Uniformat for BCA and envelope. That would eventually produce inconsistent codes and duplicated maintenance.

Instead:

- keep one master code library;

- associate codes with multiple assessment profiles;

- set different default depths and workflow requirements;

- allow PM-level scope refinement with audit controls.

The platform should therefore behave like a configuration-driven assessment engine, not a fixed checklist.

The strongest first deliverable would be an Excel-based Configuration Matrix containing every code with columns for:

- BCA applicability;

- BCA default state;

- BCA default depth;

- Envelope applicability;

- Envelope default state;

- Envelope default depth;

- field requirements;

- analyst requirements;

- costing requirements;

- output mappings;

- PM override permission;

- rationale.

That matrix would become the functional specification for the implementation team.

now if i ask you to show this selction graphicaly , how would you do it. i am thiniking about the whole Matrix, then using shadowing and colr sceem displaying the scope of BCA nd Building envelop. Thoughts

I agree with your direction. The most effective visual would be a hierarchical scope matrix using colour, shading and symbols to show where BCA and Building Envelope overlap and where they diverge.

I would not rely on colour alone. The matrix should combine:

- colour;

- shading intensity;

- labels;

- activation symbols;

- hierarchy indentation.

Recommended visual model

Rows

Each row represents a Uniformat code:

A

A10

A1010

A1010.10

A1010.10.01

B

B10

B1010

The hierarchy should be expandable and collapsible so the user can view:

- Level 1 overview;

- Levels 1–3 for scope definition;

- Levels 4–5 for detailed configuration.

Columns

| Column | Purpose |
| --- | --- |
| Code | Uniformat code |
| Element | Component description |
| Level | 1–5 |
| BCA | BCA applicability |
| Envelope | Envelope applicability |
| Default depth | Standard inspection depth |
| PM override | Whether PM can change it |
| Costing | Quantity and cost required |
| Reporting | Report output requirement |

Colour scheme

I recommend four primary scope states:

| Visual treatment | Meaning |
| --- | --- |
| Blue | BCA only |
| Orange | Building Envelope only |
| Purple | Applicable to both |
| Light gray | Not normally applicable |
| White with dashed outline | Optional or conditional |
| Dark gray | Administrative/non-inspection code |

Then use shading intensity to communicate depth:

| Shading | Meaning |
| --- | --- |
| Light | Optional or screening level |
| Medium | Default included |
| Dark | Mandatory or core scope |

For example:

- B2010 Exterior Walls would likely be dark purple because it is core to both BCA and Envelope.

- D3020 Heat Generating Systems would likely be dark blue because it is core BCA but generally outside envelope scope.

- C3030 Ceiling Finishes may be light orange or conditional because it can provide evidence of envelope leakage but is not itself an envelope system.

- H3010 Reports would be dark gray because it supports administration and reporting, not physical inspection.

Recommended two-level visualization

1. Executive scope heatmap

A high-level matrix showing only Levels 1–3.

This is used for:

- management review;

- service-line definition;

- client discussions;

- proposal scoping;

- PM project setup.

Example:

| Uniformat system | BCA | Envelope |
| --- | --- | --- |
| A10 Foundations | Core | Selective |
| A20 Basement Construction | Core | Core |
| B10 Superstructure | Core | Selective |
| B20 Exterior Closure | Core | Core |
| B30 Roofing | Core | Core |
| C10 Interior Construction | Core | Limited |
| C30 Interior Finishes | Core | Evidence only |
| D20 Plumbing | Core | Selective |
| D30 HVAC | Core | Interface only |
| G20 Site Improvements | Core | Selective |
| G30 Site Utilities | Core | Selective |

This view communicates the scope immediately.

2. Detailed configuration matrix

A complete Levels 1–5 matrix used by the implementation team and administrators.

This view contains:

- activation states;

- default inclusion;

- inspection depth;

- data requirements;

- PM permissions;

- report mapping;

- costing rules.

The executive heatmap should be generated from the detailed configuration matrix rather than maintained separately.

Recommended symbols

Colour should show the service line, while symbols show the configuration status.

| Symbol | Meaning |
| --- | --- |
| ● | Required |
| ✓ | Default included |
| ○ | Optional |
| ◐ | Conditional |
| — | Excluded |
| NP | Not present |
| OS | Out of scope |
| IA | Inaccessible |

A cell could therefore display:

● L4

Meaning:

- required;

- configured to Level 4.

Or:

○ L5

Meaning:

- optional;

- Level 5 detail is available.

Example of the visual logic

ASSESSMENT PROFILE

Uniformat Code   Component                  BCA        Envelope     Depth

─────────────────────────────────────────────────────────────────────────

A                Substructure               ■■■         ■■□          L3

A10             Foundations                ■■■         ■□□          L3

A1010          Standard Foundations       ■■■         ■□□          L3

A1030          Slab on Grade              ■■■         ■■□          L4

A20             Basement Construction      ■■■         ■■■          L4

B                Shell                      ■■■         ■■■          L4

B10             Superstructure             ■■■         ■■□          L3

B20             Exterior Closure           ■■■         ■■■          L5

B2010          Exterior Walls             ■■■         ■■■          L5

B2020          Exterior Windows           ■■■         ■■■          L5

B2030          Exterior Doors             ■■■         ■■■          L4

B30             Roofing                    ■■■         ■■■          L5

C                Interiors                  ■■■         ■□□          L3

C30             Interior Finishes          ■■□         ◐            L3

D                Services                   ■■■         ■□□          L3

D20             Plumbing                   ■■■         ◐            L3

D30             HVAC                       ■■■         ◐            L3

G                Sitework                   ■■■         ■■□          L3

The blocks represent strength of applicability:

- ■■■ Core;

- ■■□ Selective;

- ■□□ Limited;

- ◐ Conditional.

Interactive configuration concept

The same visual matrix can become the configuration screen.

The administrator would be able to:

- switch between BCA and Envelope;

- expand or collapse levels;

- activate or deactivate branches;

- set default depth;

- designate codes as required, optional or conditional;

- preview the resulting PM project template.

The PM would receive a simpler version:

- preconfigured scope;

- visible included and excluded branches;

- ability to override permitted codes;

- warnings when removing required codes;

- automatic inheritance to child codes.

Here is a compact illustration of how the interactive model could behave:

Uniformat Scope Configuration Matrix

Give feedback

My recommendation

Use three related visuals, not one oversized diagram:

- Level 1–3 service-line heatmap
Shows the overall relationship between BCA and Envelope.

- Level 1–5 configuration matrix
Shows every code, activation status, depth and rule.

- Project scope summary
Shows what the PM selected for one particular assessment.

This creates a clear progression:

Master Uniformat Matrix

↓

Service Configuration

↓

Project-Specific Scope

Your shading concept is therefore correct. The improvement is to use colour for service applicability, shading for strength of applicability, and symbols for configuration status.